| American University of Sharjah College of Engineering Dept of Computer Science & Eng P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010A Phone: 971-6-5154948 E-mail: sshapsough@aus.edu Semester: Fall 2024 |
| --- | --- | --- |

Lab #7 – Arithmetic Circuits

Objectives:

- Analyze the operation of arithmetic circuits

- Build an adder/subtractor

Lab Equipment and Circuit Components:

- Equipment

  - NI ELVIS Board

  - Connectors

- Circuit components

  - IC Type 7483/74283 4-bit Parallel binary adder

  - IC Type 7404, 7432, 7486

Introduction:

Addition

There are two main categories of adder circuits: The Half Adder and the Full Adder as shown in Figure 1. The Half Adder accepts two binary bits on its inputs and produces two binary bits on its outputs, a sum bit and a carry bit. The Full Adder accepts two input bits and an input carry and generates a sum output and a carry-out. Two Half Adders can be connected to obtain one Full Adder.

Figure  SEQ Figure \* ARABIC 1.  Logic diagram for Half and Full Adder

Figure  SEQ Figure \* ARABIC 2.  Basic 2-bit Ripple-Carry Adder

To add two numbers, a Full-Adder is required for each bit in the numbers. So, for 2-bit numbers, two adders are needed; for 4-bit numbers, four adders are used; and so on. The carry output of each adder is connected to the carry input of the next-higher order adder as shown in Figure 2. This configuration is called Ripple-Carry Adder. Notice that either a half-adder can be used for the least significant position or the carry input of a Full-Adder can be grounded (logic 0) since there is no carry input to the least significant bit position.

Subtraction

The subtraction of two binary numbers can be done by taking the 2’s complement of the subtrahend and adding it to the minuend. The 2’s complement can be obtained by taking the 1’s complement and adding 1 to it. To perform (A – B), we complement the four bits of B, add them to the four bits of A, and add 1 to the input carry. This is done as shown in Figure 3 for a 2-bit Adder/Subtractor circuit.

Two X-OR gates complement the bits of B when the mode select M=1 (because x⊕1=x’) and leave the bits of B unchanged when M=0 (because x⊕0=x) thus, when the mode select M is equal to 1, the input carry Ci is equal to 1 and the sum output is A plus the 2’s complement of B. When M is equal to 0, the input carry is equal to 0 and the sum generates A + B.

Figure  SEQ Figure \* ARABIC 3.  2-bit Adder/Subtractor

IC Solution

The IC type 7483/74283 in Figure 4, is a 4-bit binary adder with fast carry.  The two 4-bit input binary numbers are A1 through A4 and B1 through B4. The 4-bit sum is obtained from S1 through S4. Ci is the input carry and Co the out carry. This IC can be used as an Adder/Subtractor and as a magnitude comparator.

C4∑4∑3∑2∑1=A4A3A2A1+B4B3B2B1+C0

Figure 4.    7483/74283 4-bit binary adder

Lab Assignment:

Exercise 1:

Refer to the 7483/74283 datasheet and:

        - Design a circuit to add two four-bit binary numbers using the 7483/74283.

        - Design an overflow detector circuit that will turn ON an LED if the sign of the output result is invalid.

HINT: Overflow happens when

- Negative + Negative = Positive

- Positive + Positive = Negative

Implement and test the circuit by adding signed numbers and completing the following table: (negative numbers must be initially in 2’s complement format).

| Addition A+B | Binary Inputs |  | Binary Output | Overflow Indicator |
| --- | --- | --- | --- | --- |
|  | A4 A3 A2 A1 | B4 B3 B2 B1 | C4 S4 S3 S2 S1 |  |
| 7 + 7 |  |  |  |  |
| 4 + 6 |  |  |  |  |
| 6 + 5 |  |  |  |  |
| 7 + -4 |  |  |  |  |
| -1 + 6 |  |  |  |  |
| -5 + -7 |  |  |  |  |

Exercise 2:

Design an adder/subtractor circuit using 7483/74283 and 7486 chips. A control input, M, determines whether the circuit adds or subtracts two four-bit numbers using two’s complement binary arithmetic.

| Control Input (M) | Operation |
| --- | --- |
| 0 | A+B |
| 1 | A-B |

Implement and test the circuit and fill the following table:

| Control M | Decimal Number |  | Binary Inputs |  | Binary Output |
| --- | --- | --- | --- | --- | --- |
|  | A B |  | A4 A3 A2 A1 | B4 B3 B2 B1 | C4 S4 S3 S2 S1 |
| 0 | 7 | 0 |  |  |  |
| 0 | 6 | 1 |  |  |  |
| 0 | 4 | -8 |  |  |  |
| 1 | -1 | -8 |  |  |  |
| 1 | -8 | -6 |  |  |  |
| 1 | -5 | -4 |  |  |  |