| American University of Sharjah College of Engineering Dept of Computer Science and Engineering P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010A e-mail: sshapsough@aus.edu Semester: Fall 2023 |
| --- | --- | --- |

Lab #8 – Working with Decoders and Multiplexers

Objectives:

- To demonstrate the operation of decoders.

- To know how to use the decoders to implement combinational circuits.

Lab Equipment and Circuit Components:

- Equipment

  - LogiSim Evolution

Introduction:

LogiSim Evolution

LogiSim Evolution is a new branch of the LogiSim simulator, it add support for many TTL chips in addition to GUI improvements, please download it from the following link: https://drive.google.com/file/d/1Qeo63cYMRT95W1s6hUoM3bvHM0V1BAnL/view?usp=sharing

Once you download it and extract it you will get something like the following:

Decoders

The decoder is a combinational logic circuit that converts binary information from (n) inputs to maximum of (2n) unique output lines.  Any combinational logic circuit with (n) inputs and (m) outputs can be implemented with an (n-to-2n) line decoder and (m) OR gates in the case of active high decoders or (m) NAND gates in the case of active low decoders. Decoders are available in 3-to-8 (74138) 4-to-16 (74154), and dual 2-to-4 (74139) MSI chips (check data sheets). Figure 3 shows the Function Table and Connection Diagram of the 74138 IC. Note that the outputs of 74138 and 74154 are active low, i.e. only the active output is at LOW state while the rest are at HIGH state.

Figure 3.	Connection Diagram and Function Table of 74138

Figure 4.	Connection Diagram and Function Table of 74154

Figure 5.	The 74138 chip on LogSim Evolution

Lab Assignment:

Exercise 1:

A given circuit outputs a 1 when a 3-bit binary number has an odd number of 1s, and 0 otherwise. Build the circuit using the 74138 (3-to-8 decoder) chip and any necessary extra gates.

  - List the truth table of the circuit.

  - Implement the circuit on LogiSim Evolution

Exercise 2:

A given circuit accepts a 4-bit input which represents a month (e.g., input 0001 represents January). The circuit should output a one if a given month has only 30 days in it. Design the circuit using the 74138 (3-to-8 decoder) chip shown above and any necessary extra gates.

- List the truth table of the circuit.

- Implement the circuit on LogiSim Evolution

Exercise 3:

Design a one-bit full adder using 74139 (dual 2-to-4 decoder) chip and any necessary logic gates.

  - List the truth table of the circuit.

  - Implement the circuit on LogiSim Evolution