| American University of Sharjah College of Engineering Dept of Computer Science and Engineering P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010A e-mail: sshapsough@aus.edu Semester: Fall 2023 |
| --- | --- | --- |

Lab #9 – Working with Multiplexers

Objectives:

- To demonstrate the operation of Multiplexers

- To know how to use the MUXs to implement combinational circuits.

- To know how to build a 4-bit unsigned comparator.

Lab Equipment and Circuit Components:

- Equipment

  - LogiSim Evolution

Introduction:

Multiplexers

The 74151 is an 8-line to 1-line multiplexer (MUX). Its schematic representation is shown in Figure 1 (check data sheet). Selection lines C, B, and A; select the particular input to be multiplexed and applied to the output. Strobe input (S) acts as an enable signal. If S=1, the chip 74151 is disabled and output Y=0. If S=0 then the chip 74151 is enabled and functions as a multiplexer. Figure 1 shows the function table of 74151.

Figure 1.	Connection Diagram and Function Table of 74151

The 74153 is a dual 4-line to 1-line multiplexer. It has the schematic representation shown in Figure 2. Selection lines B and A select the particular input xC0, xC1, xC2, or xC3 to be multiplexed and applied to the output Yx. The enable signals Gx (x = 1 or 2) acts as an enable signal for the corresponding multiplexer. Figure 2 shows Function Table of the multiplexer 74153. Note that each of the on-chip multiplexers is independent from the other, while sharing the same select lines B and A.

Figure 2.	Connection Diagram and Function Table of 74153

Lab Assignment:

Exercise 1:

Design the following Boolean function using 74151 multiplexer and use LogiSim Evolution to simulate the circuit of question 1 using 74151 and necessary logic gate(s):

F (A, B, C, D) = ∑ (0, 4, 5, 7, 8, 9, 12, 15)

| Enable | # Select Lines |  |  | Output |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## OE | S2 | S1 | S0 | Y |  |  |
| 1 | X | X | X | 0 |  |  |
| 0 | 0 | 0 | 0 | A |  |  |
| 0 | 0 | 0 | 1 | B |  |  |
| 0 | 0 | 1 | 0 | C |  |  |
| 0 | 0 | 1 | 1 | D |  |  |
| 0 | 1 | 0 | 0 | E |  |  |
| 0 | 1 | 0 | 1 | F |  |  |
| 0 | 1 | 1 | 0 | G |  |  |
| 0 | 1 | 1 | 1 | J |  |  |
| Table (1) |  |  |  |  |  |  |

Exercise 2:

Implement the function below using a 4-to-1 MUX and any necessary logic. Draw the circuit diagram, indicate all available pins on the 74153 chip, and how they must be connected. Then use LogiSim Evolution to simulate the circuit.

F (A, B, C, D) = ∑ (0, 1, 4, 5, 7, 8, 12)

Don't care = d (2, 3)

Exercise 3:

Design a 4-bit unsigned number comparator with 3 outputs:

- O(a=b): true if a=b and false otherwise

- O(a>b): true if a>b and false otherwise

- O(a<b): true if a<b and false otherwise

      - Show your design steps

      - Implement and test your final circuit in LogiSim