| American University of Sharjah College of Engineering Dept of Computer Science & Engg P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010 Phone: 971-6-5154948 e-mail: sshapsough@aus.edu Semester: Fall 2024 |
| --- | --- | --- |

Lab #10 – Latches and D-Flip Flops

Objectives

To construct basic latches using logic gates and demonstrate their operation.

Lab Equipment and Circuit Components

Equipment

- Breadboard

- Switches and LEDs

CIRCUIT COMPONENTS (Data sheets are available in Data Sheets folder)

IC type 7400, 7402, 7404, 7474

Introduction

Latches:

A latch is a type of bi-stable logic device or multi-vibrator. An active-High input S-R (SET-RESET) latch is formed with two cross-coupled NOR gates as shown in Fig (2.a); an active-LOW input S-R latch is formed with two cross-coupled NAND gates as shown in Fig (2.b). Logic symbols for both latches are illustrated in Fig (3).

| Fig (2.a). | Fig (2.b). |  | Fig (3.a). | Fig (3.b). |
| --- | --- | --- | --- | --- |

Notice that the output of each gate is connected to an input of the opposite gate. This produces the regenerative feedback, a characteristic of all latches and flip-flop.

The gated D latch differs from the S-R latch because it has only one input in addition to EN. This input is called D (data) input. Fig (4) contains a logic diagram and logic symbol of a D latch. When the D input is HIGH and the EN input is HIGH, the latch will set. When the D input is LOW, and EN is HIGH, the latch will reset. Stated in another way, the output Q follows the input D when EN is HIGH.

Fig (4)

D Flip-flops

The 7474 chip is a Dual Positive-Edge-Triggered D-Type Flip-Flops with Preset, Clear and Complementary Outputs. The two D flip-flops are independent. The information on the D input is accepted by the flip-flops on the positive going edge of the clock pulse. The data on the D input may be changed while the clock is LOW or HIGH without affecting the outputs as long as the data setup and hold times are not violated. A LOW logic level on the preset or clear inputs will set or reset the outputs regardless of the logic levels of the other inputs. Figure 3 shows the connection diagram and function table of the 7474 D flip-flop.

Figure 3. 7474 Connection Diagram and Function Table

Lab Assignments:

Exercise 1:

Connect the circuit of Fig(2.a) and verify its operation using discrete logic gates. Fill the following truth tables for both active-HIGH S-R latches.

| R | S | ## Q | # Q’ |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 0 | 1 |  |  |
| 1 | 0 |  |  |
| 1 | 1 |  |  |

Exercise 2:

Connect the circuit of Fig(4) and verify its operation using discrete logic gates. Fill the following truth table below for the gated D-latch.

| D | EN | ## Q | # Q’ |
| --- | --- | --- | --- |
| 0 | 1 |  |  |
| 1 | 1 |  |  |
| X | 0 |  |  |

Exercise 3:

Implement and verify the operation of the D flip-flop (7474) on the Breadboard. Fill in the following table:

| Inputs |  |  |  | Outputs |  |
| --- | --- | --- | --- | --- | --- |
| PRE | CLR | CLK | D | Q | Q |
| L | L | X | X |  |  |
| L | H | X | X |  |  |
| H | L | X | X |  |  |
| H | H | ↓ | L |  |  |
| H | H | ↓ | H |  |  |
| H | H | H | X |  |  |