| American University of Sharjah College of Engineering Dept of Computer Science & Engg P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010A Phone: 971-6-5154948 e-mail: sshapsough@aus.edu Semester: Fall 2024 |
| --- | --- | --- |

Lab #11 - Counters

Objectives

- To construct basic synchronous counters and demonstrate their operation

Lab Equipment and Circuit Components

Equipment

- Breadboard

- Switches, DIP Switches, Pullup resistors and LEDs

Circuit Components

- IC type 74194, 7473, any other required IC.

Introduction

Synchronous Counters

The term synchronous refers to the events that have a fixed time relationship with each other. With respect to counter operation, synchronous means that all the flip-flops in the counter are clocked at the same time by a common clock pulse. On the clock pulse, be it rising or falling edge triggered, the output of the counter will be the increment of the previous value. In general, the procedure to design a synchronous system contains the following six main steps:

- State Diagram,

- Next-State Table,

- Flip-Flop Transition Table,

- Karnaugh Maps,

- Logic Expressions for Flip-Flop Inputs

- Final Circuit.

Lab Assignments:

Exercise 1:

Design a synchronous counter to perform odd counting from 1 to 11 and then repeat. This will require four flip-flops A, B, C, D. Use the design procedure described in the lab by applying the six steps mentioned before. Use the following J-K transition table to help you in the design.

Hint: After 11 the counter goes back to 1

The excitation table for the JK FF is given below:

| Present State | Next State | J | K |
| --- | --- | --- | --- |
| 0 | 0 | 0 | X |
| 0 | 1 | 1 | X |
| 1 | 1 | X | 0 |
| 1 | 0 | X | 1 |

| Current State |  |  |  |  | Next State |  |  |  | Flip Flops inputs |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dec | A | B | C | D | An | Bn | Cn | Dn | JA | KA | JB | KB | JC | KC | JD | KD |
| 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 0 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | 0 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 0 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 0 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 1 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 1 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 1 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |

| C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. JA=. | C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. KA=. | C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. JB=. |
| --- | --- | --- |
| C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. KB=. | C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. JC=. | C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. KC=. |
| C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. JD=. | C’D’ 0 0; C’D 0 1; CD 1 1; CD’ 1 0. A’B’ 0 0. A’B 0 1. AB 1 1. AB’ 1 0. KD=. |  |

Implement the circuit on the LogiSim to verify your solution. Use a bushbutton for the clock.

Exercise 2:

Design a synchronous counter to perform even counting from 0 to 6 and then repeat using D-FlipFlop. This will require three flip-flops D2, D1, D0. Use the design procedure described in the lab by applying the six steps mentioned before. Use the following D-FF transition table to help you in the design.

Hint: After 6 the counter goes back to 0

The excitation table for the D-FF is given below:

| Present State | Next State | D |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 1 | 1 |
| 1 | 0 | 0 |

| Current State |  |  |  | Next State |  |  | Flip Flop Inputs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dec | Q2 | Q1 | Q0 | Q2 | Q1 | Q0 | D2 | D1 | D0 |
| 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| 1 | 0 | 0 | 1 |  |  |  |  |  |  |
| 2 | 0 | 1 | 0 |  |  |  |  |  |  |
| 3 | 0 | 1 | 1 |  |  |  |  |  |  |
| 4 | 1 | 0 | 0 |  |  |  |  |  |  |
| 5 | 1 | 0 | 1 |  |  |  |  |  |  |
| 6 | 1 | 1 | 0 |  |  |  |  |  |  |
| 7 | 1 | 1 | 1 |  |  |  |  |  |  |

| B’C’ 0 0; B’C 0 1; BC 1 1; BC’ 1 0. A’ 0. A 1. D2 =. | B’C’ 0 0; B’C 0 1; BC 1 1; BC’ 1 0. A’ 0. A 1. D1 =. | B’C’ 0 0; B’C 0 1; BC 1 1; BC’ 1 0. A’ 0. A 1. D0 =. |
| --- | --- | --- |

Implement the circuit on the breadboard to verify your solution. Use a pushbutton for the clock.