| #### American University of Sharjah ## College of Engineering Dept of Computer Science & Engg P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010A Phone: 971-6-5154948 e-mail: sshapsough@aus.edu Semester: Fall 2024 |
| --- | --- | --- |

Lab #4 - Simplification of Boolean Functions Using Karnaugh Map

Objectives

- Study and demonstrate the use of basic logic gates: NAND, NOT, AND, and OR.

- To use Karnaugh map to simplify Boolean expressions.

Lab Equipment and Circuit Components

a. Equipment

    - Breadboard

    - Switches and LEDs.

    - Connectors.

b. Circuit Components

    - 7400, 7404, 7408, 7432, 7486 chips

Introduction:

Karnaugh maps or K-maps for short provide another means of simplifying and optimizing logical expressions. This is a graphical technique that utilizes a sum of product (SOP) form or product of sums.

In order to use a K-map, the truth table for a logical expression is transferred to a K-map grid. The grid for three and four input expressions are provided in the figures below. Each cell corresponds to one row in a truth table or one given state in the logical expression. The order of the items in the grid is not random at all; they are set so that any adjacent cell differs in value by the change in only one variable. Because of this, items can be grouped together easily in rectangular blocks of two, four, and eight to find the minimal number of groupings that can cover the entire expression. Note that diagonal cells require that the value of more than two inputs change, and hence are not considered neighbors and therefore cannot be combined to form larger cells.

|  | C’D’ 0 0 | C’D 0 1 | CD 1 1 | CD’ 1 0 |
| --- | --- | --- | --- | --- |
| A’B’ 0 0 |  |  |  |  |
| A’B 0 1 |  |  |  |  |
| AB 1 1 |  |  |  |  |
| AB’ 1 0 |  |  |  |  |

|  | B’C’ 0 0 | B’C 0 1 | BC 1 1 | BC’ 1 0 |
| --- | --- | --- | --- | --- |
| A’ 0 |  |  |  |  |
| A 1 |  |  |  |  |

|  | B’ 0 | B 1 |
| --- | --- | --- |
| A’ 0 |  |  |
| A 1 |  |  |

Lab Assignment

Question 1:

Given the following Boolean function where:

F (A, B, C) = ∑m (0, 1, 2, 4, 6)

- Build the Truth Table.

- Extract the Sum of Maxterms (POS) form.

- Simplify the expression in part (b), show your work.

- Simulate the simplified circuit using Logisim.

- Implement the simplified circuit on the breadboard.

Question 2:

Given the following kmaps

|  | B’ 0 | B 1 |
| --- | --- | --- |
| A’ 0 | 1 | 0 |
| A 1 | 0 | 1 |

|  | B’ 0 | B 1 |
| --- | --- | --- |
| A’ 0 | 0 | 1 |
| A 1 | 1 | 0 |

|  | B’ 0 | B 1 |
| --- | --- | --- |
| A’ 0 | 1 | 0 |
| A 1 | 1 | 0 |

F(A,B)				G(A,B)					H(A,B)

- Extract the expression for each of F, G, and H.

- Simulate all three functions in Logisim.

- Implement all three circuits on the breadboard.