| American University of Sharjah College of Engineering Dept of Computer Science and Engineering P. O. Box 26666 Sharjah, UAE |  | Lab Instructor: Salsabeel Shapsough Office: ESB-1010A Phone: +971 6 515 4948 e-mail: sshapsough@aus.edu Semester: Fall 2024 |
| --- | --- | --- |

Lab #6 – Arithmetic Circuits 1

Objectives:

- Analyze the operation of arithmetic circuits

- Build a 2-bit comparator

Lab Equipment and Circuit Components:

- Equipment

  - Breadboard

  - Pushbuttons and LEDs

  - Connectors

- Circuit components

  - IC Type 7404, 7432, 7486

Lab Assignment:

Exercise 1:

The Two’s complement of a binary number is equivalent to taking the one’s complement and then adding one. Design 4-bit combinational circuit 2’s complementor. The input is the binary number ranging from 0000 to 1111. The output is the 2’s complement of the input binary number.

- Build the truth table and design your circuit.

- Simulate your circuit.

- Implement and verify your operation.

Exercise 2:

Design a signed 2-bit number comparator which outputs true if A is bigger than B, and false otherwise.

| Signed Binary | Decimal |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | -2 |
| 11 | -1 |

- Complete the truth table and design your circuit.

| A1 | A0 | B1 | B0 | A>B |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |  |
| 0 | 0 | 0 | 1 |  |
| 0 | 0 | 1 | 0 |  |
| 0 | 0 | 1 | 1 |  |
| 0 | 1 | 0 | 0 |  |
| 0 | 1 | 0 | 1 |  |
| 0 | 1 | 1 | 0 |  |
| 0 | 1 | 1 | 1 |  |
| 1 | 0 | 0 | 0 |  |
| 1 | 0 | 0 | 1 |  |
| 1 | 0 | 1 | 0 |  |
| 1 | 0 | 1 | 1 |  |
| 1 | 1 | 0 | 0 |  |
| 1 | 1 | 0 | 1 |  |
| 1 | 1 | 1 | 0 |  |
| 1 | 1 | 1 | 1 |  |

- Simulate your circuit.

- Implement and verify your operation on Tinkercad.